# Principles to be adopted

## Frequency

### Inspections in Urban areas

Urban highways are defined as those within a 40mph or lower speed limit.

Urban areas will therefore include some villages and as well as the towns and major “urban” areas of Lancashire. The inspection frequencies set out below will be applied to the urban sections of highway in the County and are governed largely by their footway hierarchy i.e. all highway elements (including carriageways) are inspected at this frequency.

| Category (COPMM) | Location (Footway Description) | Inspection Frequency |
| --- | --- | --- |
| 1 | Primary Footway – busy urban shopping and business area footways, major pedestrian routes, some “A” road footways. | Monthly |
| 2 | Secondary Footway – medium usage footways feeding into primary footways, local shopping centres, some “B” and “C” road footways. | 3 monthly |
| 3 | Link Footway – unclassified road footways linking local access footways through urban areas. | 6 monthly |
| 4 | Local Access Footway -unclassified road footways, low usage, residential estate footways, culs-de-sac etc. | 12 monthly |
| 5 | Remote Cycleway - routes for cyclists not contiguous with carriageways or footways. | 6 monthly |

COPMM – Code of Practice for Maintenance Management

### Inspections in Rural areas

### Rural highways are defined as those with a speed limit greater than 40mph.

The inspection frequencies set out below will be applied to the rural sections of highway in the County and are governed largely by their carriageway hierarchy i.e. all highway elements (including footways) are inspected at this frequency.

| Category (COPMM) | Location (Carriageway Description) | Inspection Frequency |
| --- | --- | --- |
| 1 | Motorway – Principal Motorways M65(P), A601(M) | Weekly |
| 2 & 3a | Strategic Route / Main Distributor – mainly principal “A” roads linking urban centres to strategic network. | Monthly |
| 3b | Secondary Distributor – mainly “B” and “C” roads linking larger towns and villages with the strategic network. | 3 monthly |
| 4a | Link Road – mainly unclassified roads linking main and secondary distributor network, roads linking smaller villages with the distributor network. | 6 monthly |
| 4b | Local Access Road – unclassified roads serving limited number of properties and small settlements, carrying only access traffic. | 12 monthly |
| 5 | Remote Cycleway - routes for cyclists not contiguous with carriageways or footways. | 6 monthly |
| 6 | Remote Footway - routes for pedestrians not contiguous with carriageways. | 12 monthly |

COPMM – Code of Practice for Maintenance Management